LibriSense

An Adaptive Smart Library

— Project Proposal —

Smart Cities and the Internet of Things  ·  Practical Project 2026

M.Sc. Autonomous Systems  ·  Universität Stuttgart

Domain: Smart Building  ·  Functionality: Monitoring + Automation

Team: Dennis Gunt · [member 2] · [member 3]

Submission date: 1 May 2026

# 1 · Executive Summary

Libraries are still the silent infrastructure of deep work — but they have not kept pace with what we now know about cognitive performance, indoor-air quality, and sustainable building operation. LibriSense is an IoT-plus-AI system that turns an ordinary reading room into an environment that actively collaborates with its learners.

The system continuously monitors temperature, humidity, illuminance, noise, occupancy, and a simulated CO₂ proxy, and combines those streams with each learner's focus-session state. An AI planner (PDDL, solved with Fast Downward) decides — every few seconds — which of six actuators to move next: it dims or brightens a reading lamp, switches heating/ventilation, or escalates a break reminder from a soft dashboard nudge to a buzzer.

The design directly satisfies all six Smart-Cities course requirements: modular architecture, indirect integration over MQTT, a rich IoT fabric of seven sensors and six actuators spanning physical + simulated, automatic PDDL problem generation with a real planner in the loop, a live dashboard that renders both environment state and the current plan, and a distributed deployment across an edge Raspberry Pi and a core processing node.

This proposal presents the system focus, architecture, module-by-module design, PDDL sketch, IoT device map, timeline, and risk analysis. All deliverables are on track for the 14 July report and 20 July demo.

# 2 · Proposal-form Content (copy-paste ready)

The three tables below map one-to-one to the fields of the official Project_Proposal_Form.pdf. Their wording is written to fit the form's text boxes.

## 2.1 Project Information

| Project Information |  |  |  |
| --- | --- | --- | --- |
| 1. Student Name | Dennis Gunt | Student ID | [fill in] |
| 2. Student Name | [team member 2] | Student ID | [fill in] |
| 3. Student Name | [team member 3] | Student ID | [fill in] |
| Group ID | [assigned by course] | Project Title | LibriSense — Smart Library |

## 2.2 System Focus

| System Focus |  |  |  |
| --- | --- | --- | --- |
| Domain | ◉ Smart building ○ Autonomous vehicle Educational / public-learning space — a university or municipal library. | Type | Public educational facility |
| Functionality | Monitoring: continuous tracking of temperature, humidity, illuminance, noise level, occupancy, indoor-air-quality proxy (CO₂), and per-learner focus-session state. Automation: closed-loop control of reading-lamp brightness, heating/fan, ventilation relay, and multi-modal break reminders (dashboard → LED → buzzer → TTS) — all decided by an AI planner. |  |  |
| Components | 1. IoT Sensor Hub — Raspberry Pi 3 B+ + Sense HAT (temperature, humidity, pressure, IMU) 2. Ambient Sensing Suite — GrovePi Light + Sound sensors, PIR motion, simulated CO₂ 3. Software Sensors — Learner-session state API, Pomodoro timer, weather API 4. Smart Actuators — Plugwise smart switches (lighting, heating), Grove Relay (ventilation) 5. AI Planning Core — PDDL domain + problem generator + Fast Downward planner 6. Visualisation & UX — React/WebSocket dashboard, break-reminder escalation, plan timeline |  |  |
| Objective | Transform a traditional library into an adaptive learning environment that autonomously optimises for four competing objectives: (i) learner focus and cognitive well-being — enforcing evidence-based break rhythms (Pomodoro 25/5, long break every 4 cycles); (ii) physical comfort — regulating illuminance, temperature, and CO₂ within ergonomic ranges; (iii) energy efficiency — switching off actuators in unoccupied zones and re-using passive daylight/heat; and (iv) respect for privacy and user agency — every automated action is overridable, no biometric data leaves the edge. |  |  |

## 2.3 Project Requirements

| Project Requirements |  |
| --- | --- |
| System Design | Six strongly-decoupled modules — (1) IoT driver layer, (2) processing/state-estimation, (3) problem generator, (4) AI planner, (5) execution engine, (6) visualisation — communicate only via MQTT topics and share no in-process state, so any module can be swapped or re-deployed independently. |
| System Integration | Indirect communication through a Mosquitto MQTT broker. Topics follow a namespaced convention (library/sensors/<type>, library/state, library/plan, library/actions). JSON payloads with a lightweight schema; each module is both publisher and subscriber and knows nothing about the others' implementation. |
| IoT | ≥ 7 sensors (physical + software + simulated): Sense-HAT temperature, humidity, pressure; GrovePi light, sound; Grove PIR motion; software sensors for learner-session state, weather API, and a CO₂ proxy derived from occupancy × time. ≥ 6 actuators: Plugwise smart switch for reading lamp; Plugwise smart switch for space heater / fan; Grove Relay for ventilation; GrovePi LED + buzzer for break reminder; software actuator for TTS audio; software actuator for dashboard alert banner. |
| AI Planning | PDDL 2.1 domain with numeric fluents (temperature, illuminance, session-minutes, energy-budget) and durative actions (adjust_light, adjust_heating, ventilate, suggest_short_break, suggest_long_break, enable_focus_mode). Problem instances are auto-generated by a Python/Jinja2 generator from the current processed state. Goals express comfort ∧ break-hygiene ∧ minimum-energy as a weighted metric. Planner: Fast Downward (seq-opt-lmcut) with ENHSP as fallback for numeric planning. |
| Visualisation | Browser dashboard (React + WebSocket) shows live environment state on a library floor-plan heatmap, sensor-trend charts for the last 24 h, a Pomodoro timer, and — critically — the currently-executing plan as a Gantt-style timeline with predicate diffs (satisfies both "current state" and "latest plan" requirements). |
| System Distribution | Machine #1 — Raspberry Pi edge node: sensor/actuator drivers + MQTT client. Machine #2 — laptop / second Pi: Mosquitto broker, processing, problem generator, planner, execution, dashboard backend. Optional Machine #3 — wall-mounted kiosk running the dashboard for library users. All nodes join over the same Wi-Fi / LAN. |

# 3 · System Architecture

The diagram below is a logical view — it describes what the system does and how modules communicate, independently of the eventual deployment. The three colour bands indicate physical-machine boundaries; satisfying the "≥ 2 machines" requirement is achieved by running the Edge (blue) and Core (yellow) layers on two separate Raspberry Pis / hosts, with the optional Visualisation kiosk (purple) as a third node.

Figure 1 — LibriSense three-layer logical architecture.

## 3.1 Data-flow walk-through

- Sensors on the edge node are sampled at 1 Hz; drivers normalise readings to SI units and publish to library/sensors/<type>.

- The processing module subscribes to all sensor topics, keeps a sliding-window state, runs a small finite-state machine per learner (idle → focused → break-due → on-break), and publishes a compact world-state snapshot on library/state.

- A problem generator watches library/state and, whenever a significant change is detected (or every 30 s), materialises a PDDL problem file using Jinja2 templates.

- The planner is invoked as a subprocess; its resulting plan is published on library/plan.

- The execution module subscribes to library/plan, dispatches the next action to library/actions, and monitors the world state for drift — triggering a re-plan if reality diverges from the plan's expected effects.

- The edge node subscribes to library/actions, translates each action into a device-specific command, and actuates.

- The dashboard subscribes to library/state and library/plan to render live state + plan Gantt, and publishes user overrides back on library/overrides.

# 4 · Modular System Design

Modularity is enforced by two rules: (1) every module communicates only through MQTT topics; (2) every module is independently unit-testable with a recorded-message fixture. The table below specifies the six modules.

## 4.1 Module responsibilities

| Module | Responsibility | Tech / language |
| --- | --- | --- |
| 1 · IoT Layer | Read sensors, command actuators, publish/subscribe MQTT | Python 3, grovepi, sense-hat, paho-mqtt, plugwise |
| 2 · Processing | State aggregation, learner FSM, comfort index, CO₂ proxy | Python 3, numpy, sqlite/influxdb |
| 3 · Problem Generator | State → PDDL problem via Jinja2 templates | Python 3, jinja2 |
| 4 · AI Planner | Solve PDDL problem, emit plan JSON on MQTT | Fast Downward (C++) wrapped by Python subprocess |
| 5 · Execution | Plan dispatch, drift detection, re-plan trigger | Python 3, asyncio |
| 6 · Visualisation | Browser dashboard, WebSocket bridge, overrides | React + Recharts, FastAPI, socket.io |

# 5 · IoT Inventory

## 5.1 Sensors (≥ 4 required — 8 provided)

| Sensor | Source / Bus | Measured variable | Role in planning |
| --- | --- | --- | --- |
| Temperature | Sense HAT (I²C, physical) | °C, 0.1 K resolution | Drives heating / fan action |
| Humidity | Sense HAT (I²C, physical) | % RH | Comfort-index calculation |
| Illuminance | GrovePi light sensor | lux | Lamp dim / turn-on decisions |
| Sound level | GrovePi sound sensor | dB(A) proxy | Detects disturbance → ventilate / dim |
| Motion / occupancy | Grove PIR (I²C) | Boolean presence | Gates all actuators per zone |
| CO₂ (simulated) | Software — derived | ppm proxy | Triggers ventilation action |
| Learner-session state | Software (REST to UI) | focus-minutes, phase | Primary trigger for break actions |
| Weather API | Software (OpenWeather) | outdoor °C, cloud cover | Informs heating pre-conditioning |

## 5.2 Actuators (≥ 4 required — 6 provided)

| Actuator | Interface | Effect | Planner action |
| --- | --- | --- | --- |
| Reading lamp | Plugwise smart switch (ZigBee) | Light on / off, brightness via PWM-dimmable bulb | adjust_light(zone, level) |
| Space heater / fan | Plugwise smart switch (ZigBee) | Heating on / off | adjust_heating(zone, +/-) |
| Ventilation relay | Grove Relay (I²C) | Opens/closes simulated window / fan | ventilate(zone) |
| Break LED + buzzer | GrovePi LED + buzzer | Visual + audible alert | suggest_short_break(user) |
| TTS audio | Software — pyttsx3 / Piper | Spoken reminder via speaker | suggest_long_break(user) |
| Dashboard banner | Software — WebSocket to UI | Visual on-screen nudge | nudge(user, message) |

Device-preference bid (submit by 1 May): (A) GrovePi set — primary sensor platform, (B) Plugwise set — primary actuator platform, (C) Raspberry Pi Sense HAT — backup sensor platform, (D) Grove Relay — ventilation actuator. If an item is not awarded, the risk table in §8 lists our fallback.

# 6 · AI Planning Detail

A PDDL 2.1 domain with numeric fluents captures both the physical environment and the learner's cognitive state. The excerpt below is a sketch; the final domain will include energy accounting, hysteresis thresholds, and a metric for multi-objective optimisation.

### 6.1 Domain sketch

(define (domain librisense)

(:requirements :typing :numeric-fluents :durative-actions)

(:types zone user device)

(:predicates

(occupied ?z - zone)

(lights-on ?z - zone)

(heating-on ?z - zone)

(ventilating ?z - zone)

(session-active ?u - user)

(break-due ?u - user))

(:functions

(temperature ?z - zone)

(illuminance ?z - zone)

(co2 ?z - zone)

(session-minutes ?u - user)

(energy-used))

(:durative-action adjust_light

:parameters (?z - zone)

:duration (= ?duration 2)

:condition (and (at start (occupied ?z))

(at start (< (illuminance ?z) 300)))

:effect (and (at end (lights-on ?z))

(at end (increase (illuminance ?z) 250))

(at end (increase (energy-used) 0.06))))

(:durative-action suggest_short_break

:parameters (?u - user)

:duration (= ?duration 1)

:condition (and (at start (session-active ?u))

(at start (>= (session-minutes ?u) 25)))

:effect (and (at end (break-due ?u))

(at end (assign (session-minutes ?u) 0)))))

### 6.2 Problem generation

- Input: current library/state snapshot (JSON).

- Process: Jinja2 templates instantiate objects (zones, users), initial-state fluents (numeric values), and a goal clause derived from policy.

- Goal: (and (forall (?u - user) (not (break-due ?u))) (forall (?z - zone) (and (>= (illuminance ?z) 300) (<= (temperature ?z) 24) (>= (temperature ?z) 20)))).

- Metric: (minimize (+ (* 0.4 (energy-used)) (* 0.6 (total-time)))).

### 6.3 Planner & performance

Primary planner: Fast Downward with seq-opt-lmcut for optimal plans on small problems; fallback: ENHSP for numeric-heavy instances. Plan horizon is capped at 10 min of simulated time; re-planning is triggered on (i) new goal condition violation, (ii) 30 s timer, or (iii) observed divergence > 10 %.

# 7 · Visualisation & UX

## 7.1 Dashboard layout

- Floor-plan heatmap — per zone: °C, lux, CO₂, occupancy dots.

- Trend panel — 24 h stacked charts for each sensor stream (Recharts).

- Current plan — Gantt-style action queue with predicate diffs; colour-codes actions by state (pending / executing / done).

- Pomodoro focus ring — per learner, with streak and compliance rate.

- Energy tile — cumulative kWh since midnight vs. a naive always-on baseline.

- Override drawer — slider + toggle controls that publish to library/overrides; any override is respected for ≥ 2 min before the planner is allowed to counter.

## 7.2 Break-reminder escalation

The system respects user agency: reminders start soft and escalate only if ignored.

- t = 0: dashboard banner + colour cue (subtle).

- t + 60 s: GrovePi LED pulses in the learner's zone.

- t + 2 min: buzzer chirps twice (audible but brief).

- t + 3 min: TTS voice reminder, then snooze for 2 min.

- Compliance (taken / dismissed / snoozed) is logged and visualised for the demo.

# 8 · Risks & Mitigation

| Risk | Likelihood | Impact | Mitigation |
| --- | --- | --- | --- |
| Plugwise bid loss | Medium | We lose the hardware lottery for the smart-switch kit | Fall back to a Grove Relay driving a 230 V contactor on a bench bulb; functionally equivalent for the demo. |
| Physical CO₂ sensor unavailable | High | Course does not offer a CO₂ sensor in the hardware pool | Model CO₂ proxy from occupancy × time using standard ASHRAE formula; label as simulated in report. |
| PDDL solver timeout under numeric fluents | Medium | Fast Downward may time out on large numeric problems | Restrict planning horizon to 10 min lookahead, coarsen fluents, or fall back to ENHSP. |
| MQTT broker single point of failure | Medium | If broker crashes, whole system halts | Run Mosquitto with persistent queue + systemd auto-restart; edge node buffers ≤ 60 s of sensor data. |
| Break-reminder annoyance | Low | Users ignore or disable reminders | Escalation policy with snooze + user preferences; log compliance for report analysis. |
| Team-member illness near demo | Low | Single point of human failure | Pair-programming during last sprint; every module has a secondary owner. |

# 9 · Timeline & Milestones

| Date | Milestone / deliverable | Owner |
| --- | --- | --- |
| Apr 24 | Group registration & device bidding | All |
| May 1 | Proposal submission + device preferences | All |
| May 5 | IoT kit collection (Raspberry Pi + bid-won sensors) | All |
| May 6 – May 20 | Sprint 1 · IoT layer: driver code, MQTT skeleton, sensor smoke test | IoT lead |
| May 21 – Jun 7 | Sprint 2 · Processing + state store + dashboard v1 | Processing lead |
| Jun 8 – Jun 24 | Sprint 3 · PDDL domain, problem generator, planner integration | Planning lead |
| Jun 25 – Jul 6 | Sprint 4 · Execution, closed-loop re-planning, full integration | All |
| Jul 7 – Jul 13 | Hardening · edge-case testing, energy benchmark, demo script | All |
| Jul 14 | Report submission | All |
| Jul 15 – Jul 19 | Demo rehearsal, kiosk setup, risk mitigation drill | All |
| Jul 20 | Project demo | All |

We follow two-week sprints with a 15-minute Monday stand-up and a Friday demo to the group. All code lives in a GitHub monorepo with GitHub Actions running module-level tests on every push.

# 10 · Success Criteria

- Hardware up-time ≥ 95 % over the final 48-hour soak test.

- End-to-end plan latency (sensor event → actuator command) < 3 s for 90 % of events.

- Energy saving ≥ 25 % compared to an always-on baseline in the demo scenario.

- Break-reminder compliance ≥ 60 % over a 2-hour user session.

- Dashboard renders current state + current plan with ≤ 1 s lag.

- Full demo script executes reproducibly, including a simulated sensor-fault recovery.

# 11 · References & Tools

- Cirillo, F. — The Pomodoro Technique (1988).

- ASHRAE Standard 62.1 — Ventilation for Acceptable Indoor Air Quality.

- Fast Downward Planner — fast-downward.org

- ENHSP — Expressive Numeric Heuristic Search Planner.

- Eclipse Mosquitto — MQTT broker — mosquitto.org

- Plugwise / GrovePi / Sense HAT vendor documentation.

- PDDL 2.1 — Fox & Long (2003), Journal of AI Research.